# Objective

Overall objective is to minimize the cost of the pilot scale CLC system, considering a total annualized cost (TAC).

## Reduction

Costs: capital (reactor and cost of treatment of OC), fuel, pressure drop (fan)

Profits (or avoided costs): cost of CO2 avoided, heat

#### Avoided CO2 emissions

PCO2=110$CADtn=110CADtn×1 tn1000 kg×0.04401 kgmol CO2=  0.00484 $mol

Taken from Ontario law for 2026 (https://www.ontario.ca/laws/regulation/190241#BK21 Part III Section 11) also referenced in Gabriel’s ENMPC CLC paper

#### Heat

Same as below for oxidation

#### Pressure drop

ϕfan,j=QΔPηfanτiPe

#### OC

POC=520$USDtn, see below

#### Fuel

Are there any N2-free biosyngas?

From this paper the composition of their biogas is:

H2: 42.92, CO: 45.33, CO2: 11.28, CH4: 0.4, C2H4: 0.01, C2H6: 0.06 (I think it’s reasonable to neglect C2H4 and C2H6 since we don’t have kinetics for these and their fraction is very low)

Can also look at Yin’s biomass pyrolysis gas (has some N2 though)

Idea: take a ballpark figure for the cost of “bio syngas” (any syngas that is derived from biomass, whether it’s air gasification or steam gasification) and examine the optimal designs over changes in fuel composition for the baseline cost.

Note: at a glance, steam gasification (N2 free syngas) seems to be more expensive than air gasification, so unless I can find a cost figure for both types, it might not be the best comparison

Cost of syngas: 0.23 €/Nm3 (2011), [1]

Note this figure is for a syngas that has N2

Converting this to 2025 CAD and per m3, assuming 1.35 inflation rate for 2011 Euro to 2025 (based on online calculators) conversion rate CAD to euro: 0.232009€ Nm3×1.35 2025 euro1 2011 euro×1 CAD0.62 euro×273.15Tin assuming Tin is the same as Yin, (1123 K), price of fuel is 0.1217$CADm3

Checking other sources: The Ontario Energy Board lists natural gas prices between 10.5-19.7 ¢/m3, so the estimated cost above seems reasonable.

## Oxidation

Costs: capital (reactor and cost of treatment of OC), pressure drop

Profits (or avoided costs): profit of energy

### Energy

#### Sensible heat from outlet gas

Qe=ΔtkQCO2,out+CN2Cp,f(Tout-Tin)tf

Result: Average heat in W

#### Accumulated heat

Qa=ΔziLρs1-εc1-εbAcCp,s(Tlast-Tin)tf

Result: average heat in W

Profit from heat: Pe×η1×Qe+Qa×τO

η1 is the (turbine) efficiency factor (will depend on outlet reactor temperatures)

- From Spallina:

  - For temps above 1000 C, the electric efficiency of NG fired CLC plants is 47-53%

  - Although this paper is for high pressure CLC, values fall around 0.4 so I will assume a value of 0.4

  - Zerobin & Proll: 0.38-0.4 net efficiency electric for atmospheric CLC

η2 is an additional efficiency factor (assumed 0.7) to account for not all of the generated heat being “useful” to the turbine

Pe=0.157$CADkWh  [2] (mid-peak)

#### Pressure drop

ϕfan,j=QΔPηfanPe

Pe=0.1573.6e6 is the cost of electricity, converted from 0.157 CAD/kWh to CAD/J

Fan power to overcome pressure drop: QΔPη unit analysis: m3s×kgm s2=m2kgs3=W

Typical fan efficiency is 0.75 according to Prof. Bill Anderson ChE 572 course notes

Since flow rate is constant (we are not assuming any dynamic flow rate of profile, we can convert to J by multiplying by the total stage time)

#### OC

POC=520$USDtn [ [3]

According to the authors’ estimation (not detailed in the publication), the production cost of the direct granulation method is less than 520 USD/tn

Note that the cost of the OC should only be considered for one of the two stages, since they share the same reactor. From cost parameters POC=520$USDtn the paper this is taken from is from 2014, so converting this number to $USD (dec 2025) (according to the US bureau of labor and statistics 1 (2014) = 1.39 (2025)), taking US-CAD conversion rate from Jan 20th 2025 and assuming a US ton (907.185 kg) we get POC=520$2014 USDtn×1.39 $USD 2025$ USD 2014×1.38 $CAD1 $USD×1 tn907.185 kg=1.10$CADkg OC

OC Replacement rate:

##### CLC Experimental Literature

Bao et al. observed moderate deactivation over 40 cycles in a fluidized bed system. Yan et al. investigated packed bed CLC with red mud over 20 cycles.

Wang et al: tested 2 types of red mud, one was stable for 50 cycles  (FB) and the other was less stable.

Assume operating 8000 hours/year. minimum 15 mins per reactive stage (0.25 h) two purge stages per cycle (say 15 min total). 1 cycle: 0.75 h (minimum!)

Cycle time (general): 2*t + 0.25 ([t] = h)

Number of changes:80002t+0.2540

40 cycles: 40*0.75 = 30 hours

8000/30 = 267 changes of the OC

Assuming 1100 s reduction, 3600 s oxidation, 900 s purge: 6500 s per cycle

800065003600×40=110 changes

##### CLC Optimization/Design literature

- Kayden’s Optimal Design and Control paper assumes an OC lifetime of 5 years [4]. this is synthetic material (also no indication of why she assumed 5 years)

- In a technoeconomic evaluation of CLC, a Nickel based OC (having the best physical stability) is assumed to have a lifetime of 10000 h (> 1yr) in fluidized bed [5]

  - This same study mentions other experimental work (still fluidized bed) for ilmenite as a natural OC material. The experimental publication: [6]. [5] implies that this experimental work showed that ilmenite has a lifetime of 1315 h (I couldn’t find this figure from the experimental paper from my initial search). But other papers also cite 1315-1600 h lifetime (FB) for an Fe-based OC [7]. This corresponds to ~54-66 day lifetime (5-7 OC changes per year).

    - 1315 h seems to come from “After 40 h of operation the attrition rate stabilized to a value of 0.076%/h. Furthermore, XRD analysis of fines showed that they are formed only by iron oxide”: 1000.076=1315 h

- From Hong et al., the attrition rate of red mud is 0.012 and 0.008 wt%/min for different types of red mud (fluidized bed). According to the above estimation the lifetime would be 1000.008=12500min=208 h which corresponds to 39 changes of the OC (based on 8000 h operational year).

- Spallina’s CLC design papers:

  - Reactor design and operation strategies for a large-scale packed-bed CLC power plant with coal syngas:

    - this paper is not a techno-economic optimization and there’s no discussion on OC lifetime

  - Techno-economic assessment of an integrated high pressure chemical-looping process with packed-bed reactors in large scale hydrogen and methanol production:

    - This is not strictly CLC (chemical looping hydrogen/MeOH production). Suggests that cheaper OC materials (minerals) cost around $2/kg (this is consistent with my estimation for red mud)

    - Considers an OC lifetime of 2 years (does some sensitivity on this and considers the shortest lifetime to be 6 months)

- Since all of the metrics I’ve found for red mud (or ilmenite) are for fluidized bed reactors (which are far rougher on the OCs), and other packed bed studies assume 2-5 year particle lifetimes, after discussion with Luis we are going to assume an OC lifetime of 1 year.

## Capital cost estimation

#### Correlation

ϕcap=M&S280101.9D1.066H0.8022.18+Fc

[D] = ft

[H] = ft

Fc=FmFp

Fp=1.00 for pressure up to 50 psi (3.44 bar). M&S(1994) = 1050 [4]

Fm=1.00 for carbon steel. Although stainless steel has better thermal properties, the reactor will be insulated  [5]. Fm = 3.67 for stainless steel

| CEPI2025 | 796.5 | Chemical Engineering Magazine (July) (link) |
| --- | --- | --- |
| CEPI1994 | 368.1 | Chemical Engineering Magazine “Updating the CE Plant Cost Index” (link) |

#### Annualization

AF=i(1+i)n(1+i)n-1

Assume an interest rate of 7.95% and a lifetime of 20 years to be consistent with the work of Toffolo [6]AF = 0.1015

Total capital cost: ϕcap=

Annual operating cost:

- assume there is one reduction (or oxidation) reactor operating continuously for the whole operational year

- assume set period for the oxidation or reduction stage to factor how many cycles we could get in the operational year (starting with this approach)

- only considering one capital cost (single reactor)

# References

| [1] | F. Trippe, M. Fröhling, F. Schultmann, R. Stahl and E. Henrich, "Techno-economic assessment of gasification as a process step within biomass-to-liquid (BtL) fuel and chemicals production," Fuel Processing Technology, vol. 92, no. 11, pp. 2169-2184, 2011. |
| --- | --- |
| [2] | Ontario Energy Board (OEB), "Electricity rates," [Online]. Available: https://www.oeb.ca/consumer-information-and-protection/electricity-rates. [Accessed 24 March 2026]. |
| [3] | L. Chen, Y. Zhang, F. Liu and K. Liu, "Development of a cost-effective oxygen carrier from red mud for coal-fueled chemical-looping combustion," Energy & Fuels, vol. 29, pp. 305-313, 2015. |
| [4] | D. Bernal, "Optimal design and control of a catalytic distillation column. Case study: Ethyl tert-butyl ether (ETBE) synthesis column (M.Sc. Thesis)," Universidad de los Andes, Bogota, 2016. |
| [5] | J. Douglas, Conceptual Design of Chemical Processes (Appendix E), New York: McGraw Hill, 1988. |
| [6] | K. Toffolo and L. Ricardez-Sandoval, "Optimal Design and Control of a Multiscale Model for a Packed Bed Chemical-Looping Combustion Reactor," IFAC PapersOnLine, vol. 54, no. 3, pp. 615-620, 2021. |

https://biogasassociation.ca/images/uploads/documents/2021/value_of_biogas/presentations_apr/day2c_Youre-the-One-That-I-Want-MaryamAkbari.pdf